Hardware Requirements Specification for Drone

Annie & Jennifer

Version 1.0

Table of Contents

1. Executive Summary	3

1.1 Project Overview	3

1.2 Purpose and Scope of this Specification	3

2. Product Description	4

2.1 Product Context	4

2.2 User Characteristics	4

2.3 Assumptions/Risks	5

2.4 Constraints	5

2.5 Dependencies	5

3. Requirements	6

3.1 Functional and Hardware Requirements	6

3.1.1 General Functional Objectives	6

3.1.2 Propellers	7

3.1.4 Flight Controller (FC)	8

3.1.5 Camera	9

3.2 User Interface Requirements	9

3.3 Hardware Interfaces	10

3.3.1 Internal Interfaces (drone)	10

3.3.2 External Interfaces (Controller, FPV monitor)	11

4. Requirements Confirmation/Stakeholder sign-off	12

APPENDIX	12

Appendix A. Definitions, Acronyms, and Abbreviations	12

Appendix B. References	12

## 1. Executive Summary

### 1.1 Project Overview

Our product is a UAV VTOL (Vertical Take-Off and Landing) non-autonomous quadcopter drone that can be controlled remotely via a controller. It consists of four rotors with independent motors, an X-shaped frame, buzzer, a battery, FPV camera system, gyroscope and accelerometer. If time permits, the video data from the camera system will be used for more advanced image processing (ex. object detection). The gyroscope allows for precise positioning and control of flying direction.

The intended audience for this drone is anyone interested in aerospace or aviation, enthusiasts looking for a fun new hobby, or researchers who might need an aerial device for experiments. Professional filmmakers, photographers, and search and rescue teams would be highly interested in models of the drone with a camera and advanced image processing.

### 1.2 Purpose and Scope of this Specification

The purpose of this specification is to allow people to have access to an aerial device that can be flown for personal enjoyment, exploration purposes or experimental projects. Combined with a camera, it can be used to capture images without physically being present, which can be useful for elderly people and people with disabilities. Law enforcement can use the drone to surveille or monitor dangerous areas and identify any safety hazards. People who want to explore new places without actually using air transportation can also use it! Adding a GPS onto the drone will be helpful for search and rescue teams looking for a lost item or person.

We are the designers of the product and Mr.Roller is our manager. Our made-up client is a Canadian drone manufacturer that wants us to design and build a prototype for a new quadcopter drone model. The end user will be the customers who are looking to purchase a drone.

Curriculum expectations and client requirements shall be met with this project. This project will begin in September, with research and design taking place in the first two months. Building, testing, and development of the actual product will take place from late October to January. The product will be finished by late January.

## 2. Product Description

### 2.1 Product Context

This drone relates to other products by having the possibility to integrate other components to increase its functionality such as a camera for video streaming. This product includes two main pieces, the drone and the controller. They are independent and self-contained from the environment. The drone can interface with Raspberry Pi Pico, a FPV camera, an Xbox controller and Arduino,

### 2.2 User Characteristics

General Characteristics for all customer profiles:

- Thorough knowledge of safety procedures for operating a drone

- Have experience and practice flying quadcopter drones in acro mode

- Have spare parts available to replace certain components (ex. propellers) in case of crash or damage

- Know how to operate a Xbox controller

- Know how to charge a Lipo battery with correct safety precautions

Student:

- Ages 14+ can fly alone or else adult supervision is highly recommended

Other possible customers:

- Pilots

- Aerospace engineer

- Law enforcement

### 2.3 Assumptions/Risks

- Know how to charge a Lipo battery with proper safety precautions

- Safety equipment and potential drone hazards/safety training (safety glasses, gloves)

- Know how to fly a quadcopter drone

- The drone shall flown from a safe distance from other people

Equipment availability:

- Raspberry Pi Pico (RP2040)

- 4 ESCs with integrated BECs,

  - They must have a burst rating that is higher than the maximum amp draw of the motors

- 4 brushless motors

- MPU-6050 & NRF24L01

- 4S Lipo Battery

- Computer to view camera video

- Compatible Arduino* for the RC controller

- FPV camera with VTX transmitter and antenna

- UVC receiver compatible with a computer

- Active buzzer with built-in battery

- F450 frame

### 2.4 Constraints

- Streaming video requires a minimum computer RAM of 4GB - 8GB

- Users need an XBox controller to control the drone and a computer to receive the streaming video

### 2.5 Dependencies

- Software that allows a Raspberry Pi get data from the RF transceiver

- Arduino library for getting data from the XBox controller.

## 3.Requirements

### 3.1 Functional and Hardware Requirements

#### 3.1.1 General Functional Objectives

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.1.1.1 | The drone shall be VTOL (vertical take-off & landing), meaning that the drone can take off and land vertically without a runway |  | 10/09/24 | Mr. Roller |
| HRS-3.1.1.2 | The drone shall fly in rate mode/acro mode, meaning that the pilot needs to manually change the rate of orientation of all three axes | Unless there is a change in orientation made by the pilot, the drone shall continue flying in at the rate of orientation previously set by the pilot. The drone shall not go back to being parallel to the ground if the pilot lets go of the controller sticks (i.e. not horizontal or angle mode). | 10/09/24 | Mr. Roller |
| HRS-3.1.1.3 | The drone shall be able to pitch and roll forward and backwards as well as yaw left and right according to the mechanical input device | The orientation will be controlled through an Xbox controller | 10/09/24 | Mr. Roller |
| HRS-3.1.1.4 | The drone shall have a self-powered active buzzer for locating the drone in case it crashes or gets lost | The buzzer can be set off via a button on the RC transmitter/controller if the battery is connected. | 10/09/24 | Mr. Roller |
| HRS-3.1.1.5 | The drone shall have FPV camera system, where the video can be seen real-time on a computer |  | 10/09/24 | Mr/Roller |
| HRS-3.1.1.6 | (Optional: Each propellor arm shall be lined with an LED strip as decoration and for easy identification of the drone in case it crashes) |  | 10/09/24 | Mr. Roller |
| HRS-3.1.1.7 | (Optional: The drone shall have an ultrasonic sensor to detect any near obstacles to avoid collisions) |  | 10/09/24 | Mr. Roller |

#### 3.1.2 Propellers

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.1.2.1 | The drone shall have 4 wings/propellers |  | 10/09/24 | Mr. Roller |
| HRS-3.1.2.2 | Each wing shall operate via a brushless motor which is connected to a 2-blade propeller |  | 10/09/24 | Mr. Roller |
| HRS-3.1.2.3 | Each motor shall be controller via an ESC with an integrated BEC | The ESCs supply three-phase power to each motor | 10/09/24 | Mr. Roller |

3.1.3 Mechanical Chassis/Drone Frame

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.1.3.1 | The drone shall use a F540 frame | Wheelbase: 450mm Max take-off weight: approx. 1.8kg All internal interfaces between components will be attached to the drone frame The drone frame comes with a PDB | 10/09/24 | Mr. Roller |

#### 3.1.4 Flight Controller (FC)

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.1.4.1 | The FC shall be built via a Raspberry Pi Pico (RP2040) and coded using Micropython |  | 10/09/24 | Mr. Roller |
| HRS-3.1.4.2 | The drone shall use a MPU-6050 as the Inertial Measurement Unit(IMU) to detect its movement. The FC will be connected to the IMU | The MPU-6050 includes a gyroscope and accelerometer. It is a widely used motion-tracking sensor with 6 degrees of freedom | 10/09/24 | Mr. Roller |
| HRS-3.1.4.3 | The FC shall calculate how much thrust should be applied to each motor to achieve the orientation on the three axes based on the pilot’s input | It shall adjust the thrust based on the PID calculation with the gyroscope and accelerometer values | 10/09/24 | Mr. Roller |
| HRS-3.1.4.4 | The FC shall communicate with the 4 ESCs to control the amount of thrust applied to each individual motor | The ESCs will act as the bridge between the motors and the FC. Each ESC will have an integrated BEC to step-down the battery’s voltage | 10/09/24 | Mr. Roller |
| HRS-3.1.4.5 | The FC shall receive the pilot’s input from the on- board RC receiver | The radio receiver receives signals via a RF Transceiver (NRF24L01) | 10/09/24 | Mr. Roller |
| HRS-3.1.4.6 | The FC shall be powered through one of the ESCs | Since the ESCs will step down the voltage from the battery to 5V, the Raspberry will use the ESC as a voltage regulator instead of an external step-down converter | 10/09/24 | Mr. Roller |

(Optional/In case Raspberry Pi FC doesn’t work: Use a Speedybee F405 FC or an Arduino Micro)

#### 3.1.5 Camera

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.1.5.1 | The FPV camera shall be able to stream video to a computer via a VTX transmitter and antenna | The FPV camera we shall be purchasing comes integrated with the VTX transmitter and antenna | 10/09/24 | Mr. Roller |
| HRS-3.1.5.2 | (Optional: There shall be a button to enable and disable the stream of video) |  | 10/09/24 | Mr. Roller |
| HRS-3.1.5.3 | (Optional: The video data shall be analyzed and be able to detect objects) | Image processing would be done through the computer | 10/09/24 | Mr. Roller |

#### 3.2 User Interface Requirements

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.2.1 | Users shall control the orientation of the drone via a remote controller/radio controller that includes a RF transceiver module (NRF24L01), an XBox controller, and an Arduino board that connects the two parts together | It will communicate to a receiver via a 2.4GHz radio transmission. The receiver for the RF transceiver will be attached to the drone to receive commands | 10/09/24 | Mr. Roller |
| HRS-3.2.2 | The drone shall have an on and off switch, located on the drone frame Optional: It shall also have an emergency button to “kill” the drone if necessary | “kill” = stop the program from running | 10/09/24 | Mr. Roller |
| HRS-3.2.3 | The camera shall stream the video data to a computer for viewing | The computer will get video data through a UVC receiver | 10/09/24 | Mr. Roller |
| HRS-3.2.4 | The drone shall have a programming/debugging port for reprogramming the Raspberry Pi. | Via micro-USB B port | 10/09/24 | Mr. Roller |
| HRS-3.2.5 | Users shall be able to wirelessly control/program the Raspberry Pi via SSH from an external network. |  | 10/09/24 | Mr. Roller |

### 3.3 Hardware Interfaces

Any communication interfaces and necessary protocols that exist in your  hardware system, both internal and external.

#### 3.3.1 Internal Interfaces (drone)

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.3.1.1 | The Raspberry Pi shall communicate with the ESCs using port GP 2, 28, 15, 16 through PWM |  | 10/09/24 | Mr. Roller |
| HRS-3.3.1.2 | SSH network shall be used to wirelessly upload code onto the Raspberry Pi |  | 10/09/24 | Mr. Roller |
| HRS-3.3.1.3 | The FC shall receive the pilot’s input from the on-board RF transceiver (NRF24L01) via GP17, GP14, GP6, GP7, and GP4 |  | 10/09/24 | Mr. Roller |
| HRS-3.3.1.4 | The IMC(MPU-6050) shall receive and send signals via the GP13 (SCL) and GP 12 (SAD) using the I2C protocol |  | 10/09/24 | Mr. Roller |
| HRS-3.3.1.5 | The buzzer shall receive signals via GP3 |  | 10/09/24 | Mr.Roller |

#### 3.3.2 External Interfaces (Controller, FPV monitor)

| Req# | Requirement | Comments | Date Rvwd | Customer Reviewed / Approved |
| --- | --- | --- | --- | --- |
| HRS-3.3.2.1 | The RF transceiver on the drone shall communicate with the RF transceiver on the controller through radio transmission. | Radio transmission shall be standard 2.4 GHz frequency RF transceiver module: On drone: NRF24L01 On controller: NRF24L01 + PA + LNA | 10/09/24 | Mr. Roller |
| HRS-3.3.2.2 | The VTX transmitter shall convert video signals from the camera to radio signals. The signals are then transmitted to the antenna. The antenna transmits the signals to the RC receiver, which is connected to the computer | Radio transmission shall use 5.8Ghz frequency The RC receiver connects to the antenna using RP-SMA female and to the computer using Type-C or micro-usb port | 10/09/24 | Mr. Roller |
| HRS-3.3.2.3 | The RF Transceiver(NRF20L04) on the controller shall receive signals from the Arduino via pin CE, CSN, SCK, MOSI, MOSO. |  | 10/09/24 | Mr. Roller |

## 4. Requirements Confirmation/Stakeholder sign-off

| Meeting Date | Attendees (name and role) | Comments |
| --- | --- | --- |
| 10/09/24 | Jennifer and Annie, Mr. Roller, AeroVironment (Drone manufacturer) | Confirmed HRS-3.1.1.1 - HRS-3.3.2.3 |

### APPENDIX

The appendix should not be considered as part of the requirements.

#### Appendix A. Definitions, Acronyms, and Abbreviations

All terms, acronyms, and abbreviations used in this document.

ESC: Electronic Speed Controller

PID Controller: Proportional – Integral – Derivative controller

IMU: Inertial Measurement Unit.

- The IMU module used in this project is MPU-6050, which includes a gyroscope and accelerometer. It is a widely used motion-tracking sensor with 6 degrees of freedom.

FC: Flight Controller/Raspberry Pi Pico (RP)

Transceiver: A combination transmitter/receiver

Gyro: Gyroscope

#### Appendix B. References

https://datasheets.raspberrypi.com/pico/pico-datasheet.pdf

https://timhanewich.medium.com/taking-flight-with-the-raspberry-pi-pico-micropython-diy-quadcopter-drone-61ed4f7ee746

https://brownspace.org/how-to-build-a-drone/

https://oscarliang.com/lipo-battery-guide/

https://www.instructables.com/How-to-Build-a-Rc-Drone-and-the-Transmitter-Using-/

https://coffeebreakpoint.com/micropython/how-to-connect-a-nrf24l01-transceiver-to-your-raspberry-pi-pico/?_sm_nck=1

https://www.hackster.io/akarsh98/diy-radio-controller-for-drone-nrf24l01-fd7867

https://github.com/AndyRids/pico-nrf24

https://www.amazon.ca/HAWKS-WORK-Quadcopter-Original-version/dp/B09YQ4G4ZZ?source=ps-sl-shoppingads-lpcontext&ref_=fplfs&smid=A2Z91JXWPM95ID&th=1

https://www.amazon.ca/VIFLY-Finder-Battery-Compatible-Standalone/dp/B07JB65HZC?th=1

https://www.amazon.ca/Wolfwhoop-Adjustable-Transmitter-Cloverleaf-Aircraft/dp/B06XB2ZRBP/ref=sr_1_1?dib=eyJ2IjoiMSJ9.eIsMrkU6SjC8_MQfoHDNvBmA809VA85xwe4Pu0nIwRFreFqa4CqopMx2IG4SgYxEi9Q8nw4xbaoEHyerDCFBmHDGVCQ_9cw9lyq1PV30IPw8-HVBTPQC009IhoLmrS-92ffrz0BCuRIveekpHz5ERkCCTgO9EIZv14I3yBGLA6jpUHEQFd7uNObXoe5-cvMSloCeW1NE6zMMCp7Ynr_zZkC96ZgegvgIhHsUsviS42PZ1bYLtVd6bx_FyTmop1LjKOf_HaMiogJhTXH_Dk7pTHTEuW_P26Nu3C7eCye2su8.wiT8kUQIom1JWp9gHZ-EM6Ft3gm0FgU8cQXxF5STVOk&dib_tag=se&keywords=fpv+video+transmitter&qid=1727978711&sr=8-1

https://www.ebay.ca/itm/255299303750?chn=ps&mkevt=1&mkcid=28&srsltid=AfmBOoqUkBGuriG6DzgPG3F1Cl21gsfWIvkkPG4W2mGXxUVZbHH4ZS7OtnI

https://www.aliexpress.com/item/1005006426977793.html?src=google#nav-specification

https://dronenodes.com/how-to-build-a-drone/

http://quadcopterforum.com/threads/need-help-deciding-whether-to-use-a-pdb-or-a-4-in-one-esc.16210/

https://www.droneblog.com/fpv-drones-vs-regular-drones/

https://www.youtube.com/watch?v=CiFRNJUUdwM

https://www.youtube.com/watch?v=5BmaKVHan_c

https://brainright.com/drone/